EE 2000 Logic Circuit Design
Semester A 2021/22

Tutorial 9

- Construct a 4 x 16 decoder (with enable input) using five 2 x 4 decoder (with enable input) modules. Show the schematic diagram neatly.

Answer:

- How many selection lines are contained in a multiplexer with 1024 inputs and one output?
Answer: log21024 = 10

What is the largest number of data inputs which a multiplexer with k input selection inputs can handle? 
Answer: 2k

For the multiplexer circuits shown in the following figure below, what are Y1(A, B, C) and Y2(A, B, C) respectively?  Please list in standard sum of products form.

- Implement the function f(a, b, c, d) = m (1, 2, 5, 7, 9, 11, 13) using:
(a)	A 8-to-1-line multiplexer and a NOT gate only
(b)	A 2-to-1-line multiplexer and minimum number of AND, OR, NOT gates 
	(Hints:  Assign variable a as selection input of the MUX, and then express f as a function of b, c, d with the help of K-map)

Answer (a) :

Answer (b) :

- Draw the PAL diagram for the following truth table.

Answer:

- Simplify the output in SOP. Minimum the number of product terms.

- Create the PAL programming table.