EE2302 Foundations of Information and Data Engineering      Semester A, 2018/19

Quiz 2 (50 min) Name:__________________________	Student ID:___________

- (5 marks) Consider the vector x=1, 2, 0, 2. Find its mean, standard deviation, and root mean square. State the relationship between them.

- (15 marks) Consider the set of real n-vectors x=x1,x2,…,xn, where n is an even number. The function f(x) is defined as the average with odd indices, minus the average of the entries with even indices. Is f(x) a linear function? If so, give its inner product representation, i.e., an n-vectors a for which fx=aTx for all x. If not, give specific x, y, α, β for which superposition fails, i.e. fαx+βy≠αfx+βf(y).

- (15 marks) Suppose you want to rotate a vector (4, 6) anti-clockwise by 30°. First, write down the rotation matrix. Then find the resultant vector after rotation.

- (20 marks) Find the best straight line fit (least squares) to the measurements

b=4 at t=-2,b=3 at t=-1,b=1 at t=0,b=0 at t=2

- (5 marks) Is the set of complex numbers under multiplication a group? Why or why not?

- Consider the group G,⊗, where G consists of all positive integers that are relative prime to 10 and ⊗ denote multiplication modulo 10.

  - (5 marks) What are the elements in G?

  - (5 marks) Determine the identity and the inverses of all elements in G.

  - (5 marks) Is G,⊗ a cycle group? If so, determine all its generators. If not, explain why.

- Consider the linear code which adds three parity bits c3,c4,c5 to two information bits u1,u2 as follows: c3=u1, c4=u1+u2, c5=u2.

  - (7 marks) List all its codewords, determine its code rate and minimum distance, and state its error correction capability.

  - (8 marks) Determine its generator matrix and parity check matrix,

  - (10 marks) Suppose the vector (1, 0, 0, 1, 0) is received. Compute the syndrome. Use the syndrome to identify the error bit, if any, Explain your reasoning.