EE 2000 Logic Circuit Design
Semester A 2021/22

Tutorial 7

- 2-to-4 Line Decoder (using 1-to-2 Line Decoder)

- Write the entity declaration for a 2-to-4 Line decoder module named LDecoder2_4 that has two input named A0 and A1, and four outputs named D0, D1, D2, D3.

(LSB) A0

(MSB) A1

D0

D1

D2

D3

- Write the architecture declaration for the above 2-to-4 Line decoder named LDecoder2_4 with an architecture name “LDecoder1_2” using 2 1-to-2 Line decoders.

| Inputs | Outputs |
| --- | --- |
| A1 A0 | D0 D1 D2 D3 |
| 0 0 0 1 1 0 1 1 | 1 0 0 0 0 1 0 0 0 0 1 0 0 0 0 1 |

D0 = A1’A0’

D1 = A1’A0

D2 = A1A0’

D3 = A1A0

(LSB) A0

(MSB) A1

Solution:

- 3-to-8 Decoder

- Write the entity and architecture declaration for a 3-to-8 decoder module named Decoder3_8 that has one 3-bit input named S and one 8-bit output named Q.

- Write the circuit named Q1 using VHDL with one 3-to-8 decoder and external gates.

Solution: