EE 2000 Logic Circuit Design
Semester A 2021/22

Tutorial 10

- Using JK FFs and with the minimum number of states to design a counter with counting sequence: 0,4,2,1,6 and repeat.  State whether the circuit is a self started circuit.

- Design a sequential circuit with two JK FFs, A & B, and two inputs, x & E. if E=0, the circuit remains in the same state regardless of the value of x.  When E = 1 and x = 1, the circuit goes through the state transitions from 00 to 01 to 10 to 11 back to 00, and repeats.  When E =1 and x = 0, the circuit goes through the state transitions from 00 to 11 to 10 to 01 back to 00, and repeats.

- Using a synchronous binary counters as shown in Figure 1, design and draw a counter to generate the following repeating sequences 2 to 9 repeatedly for a free running clock.

Figure 1.

- Write the VHDL code for a D-FF with (i) asynchronous Preset and Clear, and (ii) synchronous Preset and Clear.